Registers

A register is a group of binary cells. A register with n cells can store any discrete quantity of information that contains n bits. The state of a register is an n – tuple of 1’s and 0’s. With each bit designating the state of one cell in the register. The content of a register is a function of the interpretation given to the information stored in it. Consider, for example, a 16 – bit register with the following binary content:

11000111101001

A register with 16 cells can be in one of 216 possible states. If one assumes that the content of the register represents a binary integers, then the register can store any binary number from 0 to 216-1. For